Indoor Comfort Monitor: Room Temperature & Humidity vs. Outdoor Weather

Goal & Value Proposition

Develop an IoT pipeline to measure indoor temperature and humidity in a student room and relate them to 15-minute outdoor weather signals, enabling pattern, lag, and anomaly insights under unattended operation.

Hardware & Data Sources

- Indoor (self-collected): Heltec LoRa32 (ESP32) + SHT31 temperature/humidity sensor via I2C, powered by a wall USB adapter for 24/7 operation.

- Outdoor (public): Open-Meteo 15-minute data for White City (London): outdoor temperature, relative humidity / dew point, precipitation, and wind speed.

Sampling & Storage Strategy (Reliability-first: Local + Cloud in Parallel)

- Sampling: ESP32 reads SHT31 every 5 minutes (configurable).

- Local persistence (store-and-forward): Samples are appended to LittleFS as daily CSV logs, ensuring no data loss during network outages.

- Dual-path upload: In parallel, the node attempts near-real-time uploads to: a room PC endpoint (primary local sink), and a lightweight cloud endpoint (secondary backup sink).

- Delivery confirmation: A record/log segment is marked delivered only after HTTP 2xx acknowledgement.

- Scheduled backfill: Once per day, the ESP32 uploads any remaining unsent logs to both sinks, then archives/deletes locally after confirmation.

- Rationale: This design improves resilience to both local network issues and cloud downtime, while remaining reproducible and low-maintenance.

Data Pipeline & Processing

- The room PC ingestion service stores uploaded logs and periodically queries Open-Meteo for the corresponding 15-minute weather series.

- Indoor and outdoor streams are aligned onto a common 15-minute timeline for analysis.

- Processing steps include:

- Data quality reporting (uptime, missingness, upload success rate, backlog size)

- Short-gap handling (optional interpolation with clear justification)

- Smoothing to reduce sensor spikes

- Feature engineering: indoor–outdoor deltas, rolling statistics, and lag features

- Anomaly flags for abrupt changes (e.g., humidity jumps or temperature drops)

Analytics Plan & Expected Insights

- Descriptive patterns: day/night cycles and weekly trends of indoor temperature/humidity.

- Indoor–outdoor relationships: correlations and lagged response analysis between outdoor weather variables and indoor conditions, reflecting thermal inertia and ventilation effects.

- Event/anomaly interpretation: linking indoor abrupt changes to plausible external drivers (e.g., precipitation periods) or inferred indoor events (e.g., ventilation).

Hi

The project sounds great, and good data sources if you manage to collect it all well

I guess you’d want to consider how to assess the “comfort” in this project/scenario and how to evaluate the usefulness of the data collected in your analysis

Best wishes

Thank you very much for your encouraging feedback! After your helpful prompt, I have reflected carefully on the two questions you raised — how to assess “comfort” in this scenario and how to evaluate the real-world usefulness of the collected data — and I am truly grateful for your guidance.

Comfort assessment (explicit outcome):
I will define a 15-minute Comfort Index from indoor temperature and relative humidity against an explicit comfort zone. This provides a clear outcome variable for the project, reported as time-in-comfort (%) and the frequency/duration of discomfort episodes across the collection period.

Usefulness of the data (real-world value + evaluation):
To ensure the data is useful in a practical, day-to-day sense (not just passive charts), I will implement a forward-looking Comfort Forecast & Action Planner in the dashboard. Using the indoor sensor stream together with Open-Meteo forecasts, the system will estimate short-horizon indoor humidity/comfort risk and surface actionable prompts ahead of time. For example, if the forecast indicates a sustained rainy period next week, the system will flag elevated indoor humidity risk and recommend preparing mitigation steps such as running a dehumidifier to reduce damp/mould risk; conversely, if the forecast suggests a hot and very dry period, it will flag dryness risk and prompt appropriate preparation (e.g., hydration-related self-care).

I will evaluate usefulness by (i) back-testing whether incorporating outdoor forecasts and lag modelling improves short-horizon comfort risk prediction compared with an indoor-only baseline, and (ii) assessing whether the alerts are actionable and aligned with improved comfort outcomes (higher time-in-comfort and shorter discomfort episodes) over the data collection window. In parallel, I will report end-to-end data collection quality (uptime, missingness, backlog/upload success) to demonstrate the dataset is fit for analysis.

I will also explicitly discuss key trade-offs (local vs cloud reliability/latency/maintenance) in the final report.